AUTOMATIC NOTCH FILTER

Copyright 1997 by Brian Beezley, K6STI

All Rights Reserved

Automatic Notch Filter.........................1

Connecting the Sound Card......................1

Determining Input Gain.........................2

Hotkeys........................................2

Transmit Bypass................................3

Special Options................................3

Use in Windows.................................4

Copyright and License..........................5

---- AUTOMATIC NOTCH FILTER ------------------------------------

NOTCH.EXE is a simple but effective automatic notch

filter for SSB reception.  NOTCH uses digital signal processing

but doesn't require DSP hardware.  Instead it uses the computing

power of your 486DX or better CPU and math coprocessor, and the

analog input/output capabilities of your computer's sound card.

NOTCH can greatly reduce the level of one or more interfering

tones.  It automatically determines tone frequency.  NOTCH is

almost completely transparent to speech signals.

NOTCH is a DOS program that terminates and stays

resident.  It processes audio in the background while other

programs run.  You can use special key combinations to control

NOTCH while using other software.  NOTCH requires just 6 kB of

memory.

---- CONNECTING THE SOUND CARD ---------------------------------

NOTCH can use any 16-bit Creative Labs sound card, but

it must be the genuine thing--"compatible" cards won't work.

NOTCH directly accesses the sound card and doesn't require

device drivers or the BLASTER environment variable.

Connect receive audio to the sound-card left-channel

line input.  Connect the left-channel line output to an

amplified speaker with volume control and headphone jack.

Alternatively, you can connect the sound-card speaker output to

a speaker or headphones, but you must then use your keyboard to

control listening volume.  (The miniature stereo phone-plug tip

carries the left channel.)

CAUTION:  Some amplified speakers do not use series

resistors in their headphone circuits.  These units can generate

very high sound-pressure levels when driving low-impedance

headphones.  To reduce headphone output, feed the parallel-

connected left and right headphone leads through a 51-ohm

resistor.  Always use this resistor when connecting low-

impedance headphones to the sound-card speaker output.

You can obtain receive audio from your transceiver's

speaker output, but generally it's better to use a fixed-level

audio output when available.  A fixed output level makes it

unnecessary to set your transceiver's AF-gain control for proper

sound-card drive.  (However, some transceivers don't provide

transmit monitor audio on their fixed-level output.)  If your

fixed-level output can't fully drive the line input as described

in the next section, use your transceiver's speaker output and

turn up the AF gain.

1

---- DETERMINING INPUT GAIN ------------------------------------

Type NOTCH at the DOS prompt.  NOTCH displays an error

message if it can't run.  Set your transceiver's RF gain to

maximum and tune in a strong SSB signal.  Adjust input gain by

pressing the # and # keys.  Gain changes in 2-dB steps (8-dB

steps for older Vibra 16 sound cards).  The bar graph displays

peak input amplitude.  Adjust input gain so that voice peaks

occur in the green part of the graph.  If the graph turns red,

you've overdriven the sound card and the A/D converter has

clipped.  Infrequent, brief clips aren't audible, so don't worry

if the graph occasionally flashes red.  However, if you

consistently overdrive the card, you'll hear distorted sound.

When you've set the input gain to your satisfaction,

press Esc to return to DOS.  NOTCH reports the input-gain

setting required for program installation, for example, "Enter

NOTCH 60 to install."  To install NOTCH for background signal

processing, just type NOTCH 60 (in this case) or put the command

in your AUTOEXEC.BAT file.  To uninstall NOTCH, type NOTCH U.

Unless you change the input level, you won't need to

determine input gain again.

---- HOTKEYS ---------------------------------------------------

Once you've installed NOTCH in memory, it processes

audio in the background.  You can then run another program.

NOTCH monitors the keyboard for special key combinations that

use the Ctrl and Shift keys pressed together with a third key.

It looks for these combinations:

Ctrl-Shift-N    Engage Filter

Ctrl-Shift-B    Bypass Filter

Ctrl-Shift-#    Raise Volume

Ctrl-Shift-#    Lower Volume

The notch filter is engaged when you install NOTCH.

You'll probably want to leave it that way most of the time.

Bypass the filter to evaluate its effectiveness and to detect

any speech artifacts the filter may introduce.

The Ctrl-Shift-# and Ctrl-Shift-# keys adjust output

gain (this is not the same as the input gain you previously

determined).  Adjust for comfortable listening volume.  If you

use an amplified speaker with volume control, you'll probably

find it more convenient to simply turn the control knob.  It's

better to control volume by these methods than to alter it by

adjusting transceiver audio output.  Once you've set input gain,

dynamic range degrades if you reduce the input level.

2

---- TRANSMIT BYPASS -------------------------------------------

NOTCH requires a few dozen milliseconds for audio

processing.  Normally you won't notice this short signal delay.

However, if you monitor your transmit audio, you may find a

delay even this short somewhat disconcerting.

NOTCH can automatically bypass DSP during transmit by

monitoring your transceiver's relay-control line.  Bypass

eliminates all processing delay.  To use transmit bypass,

connect a transmit contact-closure to ground to pin 2 of the 15-

pin game-port connector on your sound card.  Ground return is

pin 4 (you probably won't need to connect it).  If the contact-

closure also keys an amplifier relay, use a diode to avoid

zapping the game-port input circuit.  Connect the diode in

series with the contact-closure and the game-port input pin;

point the diode away from the pin.  The sound-card JYEN pins

must be jumpered (factory default) to enable its game port.

Disable any other game port in your computer.

When you start NOTCH, add B anywhere on the command

line, for example, NOTCH 60B.  This tells NOTCH to pay attention

to the bypass signal.

---- SPECIAL OPTIONS -------------------------------------------

NOTCH doesn't require much computational power by

today's standards, but if run on an older computer, processed

audio may become choppy or software run with it may become

unresponsive.  To reduce the computational load, start NOTCH

with C anywhere on the command line.  The resulting filter won't

notch tones as effectively, but it may run on an older computer

when the default filter won't.

Processing artifacts introduced by NOTCH usually are

inaudible.  However, persons differ in aural acuity and in their

tolerance for speech artifacts.  To reduce processing artifacts,

add A anywhere on the NOTCH command line.  This option reduces

artifacts by lowering the filter adaptation gain, but it also

degrades notch speed and depth.  Artifacts are most easily

noticed when using headphones.

You can set the output gain when you install NOTCH.

This is handy when the value you need is much different than the

default of 50.  Just place a two-digit number on the command

line anywhere after the input-gain number.

Finally, add D to the command line to use Ctrl-Alt

instead of Ctrl-Shift for control functions.  Add E to use Alt-

Shift.  Use one of these options when the Ctrl-Shift key

combination is already used by another program.

3

---- USE IN WINDOWS --------------------------------------------

The following instructions assume that all NOTCH files

are in \NOTCH.  Open a DOS prompt and type \NOTCH\NOTCH.  Set

the input gain as described earlier, then press Esc.  Note the

reported gain setting, then type EXIT.

To install NOTCH in Windows 3.x, select a program group

for NOTCH in Program Manager.  Select File | New.  In the New

Program Object menu, select Program Item and click OK.  In the

Program Item Properties menu, under Description enter Notch

Filter.  Under Command Line enter NOTCH followed by a space, the

input-gain setting NOTCH reported, and any of the command-line

options described earlier.  Under Working Directory enter

\NOTCH.  Under Shortcut Key enter N.  Click Change Icon.  Click

OK when told that no icons are available.  In the Change Icon

menu, under File Name enter \NOTCH\NOTCH.ICO.  Click OK

repeatedly to dissolve all menus.

To install NOTCH in Windows 95, start Explorer and

navigate to \NOTCH.  Using the right mouse button, drag the

NOTCH.EXE icon to the desktop.  Click on Create Shortcut(s)

Here.  Next, right-click on the dragged icon.  Click Properties,

then click the Program tab.  Under Cmd line enter NOTCH followed

by a space, the input-gain setting NOTCH reported, and any of

the command-line options described earlier.  Click on Change

icon.  Click Browse, navigate to \NOTCH, and double-click on

NOTCH.ICO.  Click OK twice.

To run NOTCH, double-click on its icon.  The hotkeys are

active only when the focus is on the NOTCH window, so the Ctrl-

Shift key combination isn't needed.  Just press N, B, #, or #.

To switch quickly to the window from another in Windows 3.x,

press Ctrl-Alt-N.  Press Esc to terminate NOTCH and close its

window.

4

---- COPYRIGHT AND LICENSE -------------------------------------

This software is copyrighted.  It's licensed, not sold.

When you purchase a license for a software product, you're

granted the right to use the software under certain conditions;

ownership of the software remains with the copyright holder.

Licensing software is different than purchasing a tangible

object that you may use or dispose of as you wish.

This software is licensed to individuals at discount for

amateur use.  It has been provided to you on the condition that

you abide by the following:

1.  You will not use the software for professional, business,

government, military, or institutional purposes.

2.  You will not sell, rent, lend, give away, or otherwise

transfer the software to others.

Brian Beezley, K6STI       (760) 599-4962

3532 Linda Vista Dr.        0700-1800 PT

San Marcos, CA 92069        k6sti@n2.net

5